P2V Process

- Contact site and have users log off of node to be migrated and log into active node (A or B).

- Ask site if they have motorolla handhelds or or the older type of handhelds (symbol, Walmart)

- Page SCHDOUT informing of start of change.

- Configure backup NIC on physical machine to be migrated.

  - Enable File and Print Sharing for Microsoft Networks.

  - Select Enable NetBIOS over TCP/IP.

  - Configure backup NIC to auto-negotiate or 1000-full.

- Enable Vendor account on physical machine.

- Gather Public and Backup NIC IP information from physical machine (used later to re-IP VM).

- Disable virtual media on DRAC for Dell PE 2850’s  ONLY(will require reboot of physical machine)

  - Reboot server

  - At DRAC prompt press “CTRL-D” to enter the DRAC menu

    - Press “PGDN”

    - Press “E” to disable virtual media

    - Press “R” to commit changes

    - Press “escape” and select save changes and exit

- Log into physical machine to be migrated and disable Public NIC (DRAC).

- Start P2V migration.

- Select new NTS280(x)us virtual machine

- Edit settings

- Remove any “Serial” devices

- Remove any “USB” devices

- Validate that “Network Adapter 1” and “Network Adapter 2” show as Adapter Type “E1000”. If they show up as “Flexible” they will need to be removed and re-added as “E1000” adapters.

- Boot Newly Created VM

- Install VMware Tools.

- Allow time to update drivers

- Select NO to reboot prompts:

- Follow instructions on HWAccel.txt displayed on screen to correct video settings.

  - Display settings has opened Automatically

  - Click Advanced

  - Click Troubleshoot tab

  - Move slider to full

  - Click Ok Twice

  - Click Next on Open Software install window,

  - Click Finish and reboot

- Configure NIC’s – reference Terminal Server Post P2V document.

- Remove ghosted NIC’s.

  - Open command prompt

  - Type – “SET DEVMGR_SHOW_NONPRESENT_DEVICES=1”

  - TYPE – “DEVMGMT.MSC” and hit enter

  - Click on “View” menu and select “Show hidden devices”

  - Navigate down to the Network interfaces portion of the device tree. “Remove phantom NIC”

- Edit the NTS280(x)us Virtual machine

- Click on “Network Adapter 1”

  - On the right under “Network Connection” Change the label from “dedicated app” to “VM Public Network”

- Contact site to validate applications.

  - Utilize “tsadmin” application to see new incoming TS connections

    - Start > run > tsadmin

- Utilizing the DRAC, disable booting to the C: and power off physical machine that was just migrated (DRAC)

- Page SCHDOUT informing of finished change.